# BASIC	COMPUTER	ORGANIZATION	AND	DESIGN

- Instruction Codes

- Computer Registers

- Computer Instructions

- Timing and Control

- Instruction Cycle

- Memory Reference Instructions

- Input-Output and Interrupt

- Complete Computer Description

- Design of Basic Computer

- Design of Accumulator Logic

# INTRODUCTION

- Every different processor type has its own design (different registers, buses, microoperations, machine instructions, etc)

- Modern processor is a very complex device

- It contains

  - Many registers

  - Multiple arithmetic units, for both integer and floating point calculations

  - The ability to pipeline several consecutive instructions to speed execution

  - Etc.

- However, to understand how processors work, we will start with a simplified processor model

- This is similar to what real processors were like ~25 years ago

- M. Morris Mano introduces a simple processor model he calls the Basic Computer

- We will use this to introduce processor organization and the relationship of the RTL model to the higher level computer processor

# THE BASIC COMPUTER

- The Basic Computer has two components, a processor and memory

- The memory has 4096 words in it

  - 4096 = 212, so it takes 12 bits to select a word in memory

- Each word is 16 bits long

CPU	RAM

0

15	0

4095

# INSTRUCTIONS

- Program

  - A sequence of (machine) instructions

- (Machine) Instruction

  - A group of bits that tell the computer to perform a specific operation

(a sequence of micro-operation)

- The instructions of a program, along with any needed data are stored in memory

- The CPU reads the next instruction from memory

- It is placed in an Instruction Register (IR)

- Control circuitry in control unit then translates the instruction into the sequence of microoperations necessary to implement it

# INSTRUCTION FORMAT

- A computer instruction is often divided into two parts

  - An opcode (Operation Code) that specifies the operation for that instruction

  - An address that specifies the registers and/or locations in memory to use for that operation

- In the Basic Computer, since the memory contains 4096 (=

212) words, we needs 12 bit to specify which memory address this instruction will use

- In the Basic Computer, bit 15 of the instruction specifies the addressing mode (0: direct addressing, 1: indirect addressing)

- Since the memory words, and hence the instructions, are 16 bits long, that leaves 3 bits for the instruction’s opcode

Instruction Format

15  14	12 11	0

I Opcode

Address

Addressing mode

# ADDRESSING MODES

- The address field of an instruction can represent either

  - Direct address: the address in memory of the data to use (the address of the operand), or

  - Indirect address: the address in memory of the address in memory of the data to use

Direct addressing	Indirect addressing

22

457

0	ADD

Operand

## +

457

35

| 1 | ADD | 300 |
| --- | --- | --- |
| 1350 |  |  |
| Operand |  |  |

300

1350

## +

AC	AC

- Effective Address (EA)

– The address, that can be directly used without modification to access an operand for a computation-type instruction, or as the target address for a branch-type instruction

# PROCESSOR REGISTERS

  - A processor has many registers to hold instructions, addresses, data, etc

  - The processor has a register, the Program Counter (PC) that holds the memory address of the next instruction to get

    - Since the memory in the Basic Computer only has 4096 locations, the PC only needs 12 bits

  - In a direct or indirect addressing, the processor needs to keep track of what locations in memory it is addressing: The Address Register (AR) is used for this

    - The AR is a 12 bit register in the Basic Computer

  - When an operand is found, using either direct or indirect addressing, it is placed in the Data Register (DR). The processor then uses this value as data for its operation

  - The Basic Computer has a single general purpose register – the Accumulator (AC)

# PROCESSOR REGISTERS

  - The significance of a general purpose register is that it can be referred to in instructions

    - e.g. load AC with the contents of a specific memory location; store the contents of AC into a specified memory location

  - Often a processor will need a scratch register to store intermediate results or other temporary data; in the Basic Computer this is the Temporary Register (TR)

  - The Basic Computer uses a very simple model of input/output (I/O) operations

    - Input devices are considered to send 8 bits of character data to the processor

    - The processor can send 8 bits of character data to output devices

  - The Input Register (INPR) holds an 8 bit character gotten from an input device

  - The Output Register (OUTR) holds an 8 bit character to be send to an output device

# BASIC COMPUTER	REGISTERS

Registers in the Basic Computer

11	0

PC

11	0

AR

Memory 4096 x 16

15	0

IR

15	0

TR

15	0

DR

CPU

7

OUTR

0	7	0

INPR

15	0

AC

List of BC Registers

| DR | 16 | Data Register | Holds memory operand |
| --- | --- | --- | --- |
| AR | 12 | Address Register | Holds address for memory |
| AC | 16 | Accumulator | Processor register |
| IR | 16 | Instruction Register | Holds instruction code |
| PC | 12 | Program Counter | Holds address of instruction |
| TR | 16 | Temporary Register | Holds temporary data |
| INPR | 8 | Input Register | Holds input character |
| OUTR | 8 | Output Register | Holds output character |

# COMMON	BUS	SYSTEM

- The registers in the Basic Computer are connected using a bus

- This gives a savings in circuitry over complete connections between registers

# COMMON	BUS	SYSTEM

S2

Memory unit 4096 x 16

S1 S0

Address

Bus 7

Write

Read

AR	1

ALU

LD INR CLR

PC	2

LD INR CLR

DR	3

LD	INR	CLR

E

AC	4

LD	INR	CLR

INPR

IR	5

LD

TR	6

LD	INR	CLR

OUTR

LD

16-bit common bus

Clock

# COMMON	BUS	SYSTEM

Memory 4096 x 16

Read Write

Address

E	ALU

INPR

AC

L  I	C

PC

L  I	C

L  I	C

DR

L

IR	L  I	C

TR

AR	OUTR	LD

L  I	C

7	1	2	3	4	5	6

16-bit Common Bus

S0	S1	S2

Register x

AR

PC DR AC IR TR

Memory

S2 S1 S0 0	0	0

0	0	1

0	1	0

0	1	1

1	0	0

1	0	1

1	1	0

1	1	1

COMMON	BUS	SYSTEM

- Three control lines, S2, S1, and S0 control which register the bus selects as its input

- Either one of the registers will have its load signal activated, or the memory will have its read signal activated

– Will determine where the data from the bus gets loaded

- The 12-bit registers, AR and PC, have 0’s loaded onto the bus in the high order 4 bit positions

- When the 8-bit register OUTR is loaded from the bus, the data comes from the low order 8 bits on the bus

# BASIC COMPUTER	INSTRUCTIONS

- Basic Computer Instruction Format

Memory-Reference Instructions	(OP-code = 000 ~ 110)

15	14	12 11	0

I	Opcode	Address

Register-Reference Instructions	(OP-code = 111, I = 0)

15	12 11	0

0	1	1	1	Register operation

Input-Output Instructions	(OP-code =111, I = 1)

15	12 11	0

1	1	1	1

I/O operation

# BASIC	COMPUTER	INSTRUCTIONS

| Symbol | Hex Code | Description |
| --- | --- | --- |
|  | I = 0 I = 1 |  |
| AND ADD LDA STA BUN BSA ISZ | 0xxx 8xxx 1xxx 9xxx 2xxx Axxx 3xxx Bxxx 4xxx Cxxx 5xxx Dxxx 6xxx Exxx | AND memory word to AC Add memory word to AC Load AC from memory Store content of AC into memory Branch unconditionally Branch and save return address Increment and skip if zero |
| CLA | 7800 | Clear AC |
| CLE | 7400 | Clear E |
| CMA | 7200 | Complement AC |
| CME | 7100 | Complement E |
| CIR | 7080 | Circulate right AC and E |
| CIL | 7040 | Circulate left AC and E |
| INC | 7020 | Increment AC |
| SPA | 7010 | Skip next instr. if AC is positive |
| SNA | 7008 | Skip next instr. if AC is negative |
| SZA | 7004 | Skip next instr. if AC is zero |
| SZE | 7002 | Skip next instr. if E is zero |
| HLT | 7001 | Halt computer |
| INP | F800 | Input character to AC |
| OUT | F400 | Output character from AC |
| SKI | F200 | Skip on input flag |
| SKO | F100 | Skip on output flag |
| ION | F080 | Interrupt on |
| IOF | F040 | Interrupt off |

INSTRUCTION	SET	COMPLETENESS

A computer should have a set of instructions so that the user can construct machine language programs to evaluate any function that is known to be computable.

• Instruction Types

Functional Instructions

- Arithmetic, logic, and shift instructions

- ADD, CMA, INC, CIR, CIL, AND, CLA

Transfer Instructions

- Data transfers between the main memory and the processor registers

- LDA, STA

Control Instructions

- Program sequencing and control

- BUN, BSA, ISZ

Input/Output Instructions

- Input and output

- INP, OUT

# CONTROL UNIT

- Control unit (CU) of a processor translates from machine instructions to the control signals for the microoperations that implement them

- Control units are implemented in one of two ways

- Hardwired Control

  - CU is made up of sequential and combinational circuits to generate the control signals

- Microprogrammed Control

  - A control memory on the processor contains microprograms that activate the necessary control signals

- We will consider a hardwired implementation of the control unit for the Basic Computer

# TIMING	AND	CONTROL

Control unit of Basic Computer

Instruction register (IR) 15	14	13	12	11 - 0

Other inputs

Increment (INR) Clear (CLR)

Clock

4-bit sequence counter (SC)

15	14 . . . . 2 1 0

4 x 16

decoder

T15

T0

Combinational Control

logic

7

D

D0

I

3 x 8 decoder

7 6 5 4 3 2 1 0

Control

signals

# TIMING	SIGNALS

- Generated by 4-bit sequence counter and 416 decoder

- The SC can be incremented or cleared.

- Example:	T0, T1, T2, T3, T4, T0, T1, . . .

Assume: At time T4, SC is cleared to 0 if decoder output D3 is active.

D3T4: SC 0

# INSTRUCTION	CYCLE

•	In Basic Computer, a machine instruction is executed in the following cycle:

- Fetch an instruction from memory

- Decode the instruction

- Read the effective address from memory if the instruction has an indirect address

- Execute the instruction

•	After an instruction is executed, the cycle starts again at step 1, for the next instruction

•	Note: Every different processor has its own (different) instruction cycle

# FETCH and DECODE

- Fetch and Decode

S2

S1	Bus

S0

T1 T0

Memory unit

Read

7

Address

AR	1

LD

PC	2

T0: AR  PC (S0S1S2=010, T0=1)

T1: IR  M [AR],  PC  PC + 1	(S0S1S2=111, T1=1)

T2: D0, . . . , D7  Decode IR(12-14), AR  IR(0-11), I  IR(15)

INR

IR

LD

Common bus

5

Clock

# DETERMINE	THE	TYPE	OF	INSTRUCTION

Start SC  

AR    PC	T0

T1

IR    M[AR],  PC   PC + 1

T2

Decode Opcode in IR(12-14), AR  IR(0-11),	I  IR(15)

(Register or I/O) = 1

= 0 (Memory-reference)

D7

(I/O) = 1

I

= 0 (register)

(indirect) = 1

I

= 0 (direct)

T3	T3	T3	T3

Execute input-output instruction

Execute register-reference

instruction

AR  M[AR]

Nothing

SC  0

SC  0

Execute	T4 memory-reference

instruction

SC     0

D'7IT3:	AR  M[AR]

D'7I'T3:	Nothing

D7I'T3:	Execute a register-reference instr. D7IT3:	Execute an input-output instr.

SC  0

AC  0

E  0

AC  AC’ E  E’

AC  shr AC, AC(15)  E, E  AC(0) AC  shl AC, AC(0)  E, E  AC(15)

AC  AC + 1

if (AC(15) = 0) then (PC  PC+1) if (AC(15) = 1) then (PC  PC+1)

if (AC = 0) then (PC  PC+1) if (E = 0) then (PC  PC+1)

S  0 (S is a start-stop flip-flop)

r: rB11: rB10: rB9: rB8: rB7: rB6: rB5: rB4: rB3: rB2: rB1: rB0:

CLA CLE CMA CME CIR CIL INC SPA SNA SZA SZE HLT

REGISTER	REFERENCE	INSTRUCTIONS

Register Reference Instructions are identified when

-	D7 = 1, I = 0

- Register Ref. Instr. is specified in b0 ~ b11 of IR

- Execution starts with timing signal T3

r = D7 IT3	=> Register Reference Instruction Bi = IR(i) , i=0,1,2,...,11

# MEMORY	REFERENCE	INSTRUCTIONS

Symbol

AND ADD LDA STA BUN BSA ISZ

Operation Decoder

D0 D1 D2 D3 D4 D5 D6

Symbolic Description

AC  AC  M[AR]

AC  AC + M[AR], E  Cout AC  M[AR]

M[AR]  AC PC  AR

M[AR]  PC, PC  AR + 1

M[AR]  M[AR] + 1, if M[AR] + 1 = 0 then PC  PC+1

- The effective address of the instruction is in AR and was placed there during timing signal T2 when I = 0, or during timing signal T3 when I = 1

- Memory cycle is assumed to be short enough to complete in a CPU cycle

- The execution of MR instruction starts with T4

AND to AC

D0T4:	DR  M[AR]	Read operand

D0T5:	AC  AC  DR, SC  0	AND with AC ADD to AC

D1T4:	DR  M[AR]	Read operand

D1T5:	AC  AC + DR, E  Cout, SC  0	Add to AC and store carry in E

# MEMORY	REFERENCE	INSTRUCTIONS

LDA: Load to AC

D2T4:	DR  M[AR] D2T5:	AC  DR, SC  0

STA: Store AC

D3T4:	M[AR]  AC, SC  0

BUN: Branch Unconditionally

D4T4:	PC  AR, SC  0

BSA: Branch and Save Return Address

M[AR]  PC, PC  AR + 1

Memory, PC, AR at time T4

Memory, PC after execution

20

PC = 21

AR = 135

136

0	BSA	135

Next instruction

Subroutine

20

21

135

PC = 136

0	BSA	135

Next instruction

21

Subroutine

1	BUN	135	1	BUN	135

Memory	Memory

# MEMORY	REFERENCE	INSTRUCTIONS

BSA:

D5T4:	M[AR]  PC, AR  AR + 1 D5T5:	PC  AR, SC  0

ISZ: Increment and Skip-if-Zero D6T4:	DR  M[AR] D6T5:	DR  DR + 1

D6T4:	M[AR]  DR, if (DR = 0) then (PC  PC + 1), SC  0

## FLOWCHART FOR MEMORY REFERENCE INSTRUCTIONS

Memory-reference instruction

AND	ADD	LDA	STA

D0T4		D1T4		D2T4		D3T4 DR  M[AR]	DR  M[AR]	DR  M[AR]	M[AR]  AC

SC  0

D0T5               AC  AC  DR

SC  0

D1T5 AC  AC + DR

E  Cout SC  0

D2T5 AC  DR

SC  0

BUN	BSA	ISZ

D4T4	D5T4	D6T4

PC  AR SC  0

M[AR]  PC AR  AR + 1

DR  M[AR]

PC  AR SC  0

D5T5	D6T5

DR  DR + 1

D6T6

M[AR]  DR If (DR = 0)

then (PC  PC + 1) SC  0

# INPUT-OUTPUT	AND	INTERRUPT

A Terminal with a keyboard and a Printer

- Input-Output Configuration

Input-output terminal

Printer

Serial communication interface

Receiver interface

Computer registers and flip-flops

OUTR

FGO

INPR	Input register - 8 bits

Keyboard

Transmitter interface

AC

INPR	FGI

OUTR Output register - 8 bits

FGI	Input flag - 1 bit

FGO	Output flag - 1 bit

IEN	Interrupt enable - 1 bit

Serial Communications Path Parallel Communications Path

- The terminal sends and receives serial information

- The serial info. from the keyboard is shifted into INPR

- The serial info. for the printer is stored in the OUTR

- INPR and OUTR communicate with the terminal serially and with the AC in parallel.

- The flags are needed to synchronize the timing difference between I/O device and the computer

# PROGRAM	CONTROLLED	DATA	TRANSFER

-- CPU	I/O Device --

/* Input */	/* Initially FGI = 0 */ loop: If FGI = 0 goto loop

AC  INPR, FGI  0

/* Output */	/* Initially FGO = 1 */ loop: If FGO = 0 goto loop

OUTR  AC, FGO  0

FGI=0

Start Input FGI  0

loop: If FGI = 1 goto loop

INPR  new data, FGI  1

loop: If FGO = 1 goto loop consume OUTR, FGO  1

FGO=1

Start Output

AC  Data

yes

yes

FGI=0

no AC  INPR

More Character

no END

yes

yes

FGO=0

no OUTR  AC

FGO  0

More Character

no END

Clear SC

Input char. to AC Output char. from AC Skip on input flag Skip on output flag Interrupt enable on Interrupt enable off

SC  0

AC(0-7)  INPR, FGI  0 OUTR  AC(0-7), FGO  0

if(FGI = 1) then (PC  PC + 1) if(FGO = 1) then (PC  PC + 1) IEN  1

IEN  0

p:

pB11: pB10: pB9: pB8: pB7:

pB6:

INP

OUT SKI SKO ION IOF

INPUT-OUTPUT	INSTRUCTIONS

D7IT3 = p

IR(i) = Bi, i = 6, …, 11

# PROGRAM-CONTROLLED	INPUT/OUTPUT

  - Program-controlled I/O

    - Continuous CPU involvement

I/O takes valuable CPU time

    - CPU slowed down to I/O speed

    - Simple

    - Least hardware

Input

| LOOP, | SKI | DEV |
| --- | --- | --- |
|  | BUN | LOOP |
|  | INP | DEV |

Output

| LOOP, | LDA | DATA |
| --- | --- | --- |
| LOP, | SKO | DEV |
|  | BUN | LOP |
|  | OUT | DEV |

# INTERRUPT	INITIATED	INPUT/OUTPUT

- Open communication only when some data has to be passed --> interrupt.

- The I/O interface, instead of the CPU, monitors the I/O device.

- When the interface founds that the I/O device is ready for data transfer, it generates an interrupt request to the CPU

- Upon detecting an interrupt, the CPU stops momentarily the task

it is doing, branches to the service routine to process the data transfer, and then returns to the task it was performing.

- IEN (Interrupt-enable flip-flop)

  - can be set and cleared by instructions

  - when cleared, the computer cannot be interrupted

# FLOWCHART	FOR	INTERRUPT	CYCLE

R = Interrupt f/f

Instruction cycle

Fetch and decode instructions

=0	R	=1

Interrupt cycle

Store return address in location 0

M[0]  PC

Execute instructions

IEN	=0

=1

=1	FGI

=0

=1 FGO

=0

Branch to location 1 PC  1

IEN  0

R  0

R  1

- The interrupt cycle is a HW implementation of a branch and save return address operation.

- At the beginning of the next instruction cycle, the instruction that is read from memory is in address 1.

- At memory address 1, the programmer must store a branch instruction that sends the control to an interrupt service routine

- The instruction that returns the control to the original program is "indirect BUN	0"

REGISTER	TRANSFER	OPERATIONS	IN	INTERRUPT CYCLE

Before interrupt

0

Memory

After interrupt cycle 0	256

1	0	BUN	1120

PC = 1

0	BUN	1120

255

PC = 256

1120

Main Program

I/O Program

255

256

1120

Main Program

I/O Program

1	BUN	0	1	BUN	0

Register Transfer Statements for Interrupt Cycle

- R	F/F  1	if IEN (FGI + FGO)T0T1T2

 T0T1T2 (IEN)(FGI + FGO):	R  1

  - The fetch and decode phases of the instruction cycle

must be modified Replace T0, T1, T2  with	R'T0, R'T1, R'T2

  - The interrupt cycle :

RT0:	AR  0,	TR  PC RT1:	M[AR]  TR,	PC  0

RT2:	PC  PC + 1, IEN  0, R  0, SC  0

# FURTHER	QUESTIONS	ON	INTERRUPT

How can the CPU recognize the device requesting an interrupt ?

Since different devices are likely to require different interrupt service routines, how can the CPU obtain the starting address of the appropriate routine in each case ?

Should any device be allowed to interrupt the CPU while another interrupt is being serviced ?

How can the situation be handled when two or more interrupt requests occur simultaneously ?

# COMPLETE	COMPUTER	DESCRIPTION

Flowchart	of	Operations

start

SC  0, IEN  0, R  0

=0(Instruction	R	=1(Interrupt

Cycle)	Cycle)

R’T0

RT0

AR  PC

R’T1

AR  0, TR  PC

RT1

IR  M[AR], PC  PC + 1

R’T2

M[AR]  TR, PC  0

RT2

AR  IR(0~11), I  IR(15)

D0...D7  Decode IR(12 ~ 14)

PC  PC + 1, IEN  0 R  0, SC  0

=1(Register or I/O)

D7	=0(Memory Ref)

=1 (I/O) I

=0 (Register)	=1(Indir)

I

=0(Dir)

D7IT3	D7I’T3	D7’IT3	D7’I’T3

Execute I/O   Instruction

Execute RR

Instruction

AR <- M[AR]	Idle

Execute MR Instruction

D7’T4

# COMPLETE	COMPUTER	DESCRIPTION

Microoperations

Fetch Decode

Indirect Interrupt

RT0: RT1: RT2:

D7IT3:

AR  PC

IR  M[AR], PC  PC + 1

D0, ..., D7  Decode IR(12 ~ 14), AR  IR(0 ~ 11), I  IR(15)

AR  M[AR]

T0T1T2(IEN)(FGI + FGO):

RT0: RT1: RT2:

Memory-Reference

R  1

AR  0, TR  PC M[AR]  TR, PC  0

PC  PC + 1, IEN  0, R  0, SC  0

AND ADD LDA

STA BUN BSA

ISZ

D0T4: D0T5: D1T4: D1T5: D2T4: D2T5: D3T4: D4T4: D5T4: D5T5: D6T4: D6T5: D6T6:

DR  M[AR]

AC  AC  DR, SC  0 DR  M[AR]

AC  AC + DR, E  Cout, SC  0 DR  M[AR]

AC  DR, SC  0 M[AR]  AC, SC  0 PC  AR, SC  0

M[AR]  PC, AR  AR + 1 PC  AR, SC  0

DR  M[AR] DR  DR + 1

M[AR]  DR, if(DR=0) then (PC  PC + 1), SC  0

# COMPLETE	COMPUTER	DESCRIPTION

Microoperations

Register-Reference

D7IT3 = r IR(i) = Bi

r:

(Common to all register-reference instr) (i = 0,1,2, ..., 11)

SC  0

CLA CLE CMA CME CIR CIL INC SPA SNA SZA SZE HLT

Input-Output

INP OUT SKI SKO ION IOF

rB11: rB10: rB9: rB8: rB7: rB6: rB5: rB4: rB3: rB2: rB1: rB0:

D7IT3 = p IR(i) = Bi

p:

pB11: pB10: pB9: pB8: pB7: pB6:

AC  0

E  0

AC  AC E  E

AC  shr AC, AC(15)  E, E  AC(0) AC  shl AC, AC(0)  E, E  AC(15) AC  AC + 1

If(AC(15) =0) then (PC  PC + 1) If(AC(15) =1) then (PC  PC + 1) If(AC = 0) then (PC  PC + 1) If(E=0) then (PC  PC + 1)

S  0

(Common to all input-output instructions) (i = 6,7,8,9,10,11)

SC  0

AC(0-7)  INPR, FGI  0 OUTR  AC(0-7), FGO  0 If(FGI=1) then (PC  PC + 1) If(FGO=1) then (PC  PC + 1) IEN  1

IEN  0